HUNTINGTOWER SCHOOL				Design & Technology Department

# PLANT RISK ASSESSMENT & CONTROL

PLANT:   BAND  SAW

## Assessor:	D. G. Fletcher			Date:	January 2008

Purpose of Machine:  Used for cutting  curved or straight cuts in thin sheets and boards.

Method for use:  Wood pieces  must be held firmly and guided forward across the table while the saw blade is running downward.

Machine to be operated by:	      Teacher only	   VCE Students	 Students Yrs 8 / 9 / 10

Level of Supervision		       1:1 With Teacher      Ask Teacher      As required.

| HAZARD IDENTIFICATION | RISK ASSESSMENT | RISK CONTROL |
| --- | --- | --- |
| ## Entanglement  Yes  No | Long hair, tie, sleeves, jewellery may catch on the saw, wheels or guards and be drawn onto the cutting blade. | Long hair tied back; remove all jewellery. Support table should be free from debris. Wear protective coat or apron. Follow SAFE OPERATING CODE. |
| ### CRUSHING  Yes  No | Fingers could be jammed if brought into contact with rubber wheels. Loosened guard may fall onto fingers. | Ensure guards are firmly attached. Keep hands 5cm clear of blade . Keep hand clear of table when adjusting guards. |
| CUTTING, STABBING, PUNCTURING  Yes  No | Fingers or hands may be cut off if contact is made with moving blade. During adjustment or maintenance fingers may be cut by contact with blades. Broken blade may fly out and strike the hand or body. | During maintenance or adjustment power point must be switched OFF. Push piece gently against blade to ensure clean cut. Follow SAFE OPERATING CODE. Eye protection must be worn. Do not stand to the right of the machine. Push stick should be used for small pieces. |
| ### SHEARING & JAMMING  Yes  No | Body parts may be cut off if they come into contact with moving blades. Small radius cuts may cause blade to jam. | Keep hands at least 10cm from blade. Ensure guard is lowered as close to the top surface of wood as possible. Provide clearance cuts when cutting small radii. |
| ELECTRIC SHOCK  Yes  No | Cables may become damaged or wires exposed. Cable may be severed if draped under the blade, or caught into the drive wheels. | Ensure regular maintenance inspections of wiring are carried out. Ensure power cable is clear of blade path or rotating wheels. Auto cut-off switches act if doors are opened. |
| FRICTION, BURNING  Yes  No | Excessive cutting speed may cause saw to jam or break welded joint. If blade jams, belts may break, or rubber tyres may be damaged. Blade guides become very hot. | Wait until saw is at full speed, and apply gentle forward pressure. Turn OFF machine, at powerpoint removing jammed timber. Ensure blade is tracking properly between guides. |
| STRIKING  Yes  No | Small offcuts may fly out, bounce off walls, or get flung out after catching in guards. | Eye protection and leather shoes must be worn. |
| SLIPPING, FALLING, TRIPPING  Yes  No | Operator may slip in sawdust on the floor near the machine. | Ensure cutting area is free from debris. Sweep or vacuum up sawdust and offcuts on floor. Sweep away offcuts on the table only after saw has stopped. |
| ERGONOMIC  Yes  No | Excessive machine noise will cause deafness. | Hearing protection must be worn by both operator, and those working nearby. |
| DUST, FUMES, VAPOURS  Yes  No | Sawdust may build up on cutting table, floor or machine parts. | Safety glasses must be worn. Sweep up offcuts or vacuum sawdust. Vacuum clean the wheels periodically. |
| OTHER HAZARDS | Welded blades may break. Offcuts may fall into the wheel cavity. | Guide fences must be done up firmly. Do not attempt to cut very short pieces. Round dowels must be firmly held or supported in a V-block to prevent rolling. |

SHORT TERM CONTROLS

- Routine maintenance is carried out on machine, with regular inspection for defects.

- SAFE OPERATIONAL PROCEDURES are displayed, and students are aware of them.

- Training is provided for all people using machine, with evaluation of skills checked.