USING  AUDACITY

Part 1. 	Editing a .wav file using Audacity

Handout: Audacity 1.2 Tutorial Steps 1 to 7. (ref: www.edhsonline.org/other/audacity)

Part 2.	Recording using a microphone using Audacity 1.2 and Windows Volume Control.

Open the program ‘Audacity’.

Plug your microphone into the microphone input

of your computer.

In the top corner you will see a microphone and a down-arrow.

- Click on the arrow and select ‘Monitor input’

Now we need to set the microphone input volume.

- Select ‘Microphone’.

- Set this to maximum level.

The Windows Volume Control needs to be set correctly so recording can be done.

- Double click the Volume Control icon.

- Select Options, Properties

- In ‘Adjust Volume for’, select Recording.

- Tick the check box for ‘Microphone’ if it is not already ticked.

- Select ‘OK’.

You will now only see ”Recording Control’ that only has 3 selections.

Tick the ‘Select’ box for microphone and make sure the volume control is set at the maximum level.

- Minimise this box.

Turn ON your microphone.

YOU ARE NOW READY TO RECORD.

Recording.

- Click on the record button and record you voice.

- Select the stop button when complete.

If you forget to hit STOP remember you can cut the blank section off.

TASK:

In groups of three record your voice for 2 seconds each, consecutively. This will result in a 6 second recording. Have a look at your voice variations, some of your voices create more vibrations than others.

Us the effects and edit your recordings.

Part 3. 	Using Multitrack Audio to create a Music Track.

Handout: An Audacity Tutorial by Daniel James.

(ref: http://www.daniel.uklinux.net/tutorial/)

## Features

This is a list of features in Audacity.

### Recording

Audacity can record live audio through a microphone or mixer, or digitize recordings from cassette tapes, vinyl records, or minidiscs. With some sound cards, it can also capture streaming audio.

- Record from microphone, line input, or other sources.

- Dub over existing tracks to create multi-track recordings.

- Record up to 16 channels at once (requires multi-channel hardware).

- Level meters can monitor volume levels before, during, and after recording.

### Import and Export

Import sound files, edit them, and combine them with other files or new recordings. Export your recordings in several common file formats.

- Import and export WAV, AIFF, AU, and Ogg Vorbis files.

- Import MPEG audio (including MP2 and MP3 files) with libmad.

- Export MP3s with the optional LAME encoder library.

- Create WAV or AIFF files suitable for burning to CD.

- Import and export all file formats supported by libsndfile.

- Open raw (headerless) audio files using the “Import Raw” command.

- Note: Audacity does not currently support WMA, AAC, or most other proprietary or restricted file formats.

### Editing

- Easy editing with Cut, Copy, Paste, and Delete.

- Use unlimited Undo (and Redo) to go back any number of steps.

- Very fast editing of large files.

- Edit and mix an unlimited number of tracks.

- Use the Drawing tool to alter individual sample points.

- Fade the volume up or down smoothly with the Envelope tool.

### Effects

- Change the pitch without altering the tempo, or vice-versa.

- Remove static, hiss, hum, or other constant background noises.

- Alter frequencies with Equalization, FFT Filter, and Bass Boost effects.

- Adjust volumes with Compressor, Amplify, and Normalize effects.

- Other built-in effects include:

  - Echo

  - Phaser

  - Wahwah

  - Reverse

### Sound Quality

- Record and edit 16-bit, 24-bit, and 32-bit (floating point) samples.

- Record at up to 96 KHz.

- Sample rates and formats are converted using high-quality resampling and dithering.

- Mix tracks with different sample rates or formats, and Audacity will convert them automatically in realtime.