Module 11 TCP/IP Transport and Applications Layers – Review Questions

- What are the primary duties of the Transport Layer ?

- In the example of speaking a foreign language, what analogy can be used to describe reliability and flow control ?

- List some of the basic services of the Transport Layer.

- Describe the layers of the OSI model at which the TCP/IP suite operate ?

- Describe multiplexing.

- Describe a typical handshake sequence as two hosts prepare to transfer data from one to the other or to establish a connection.

- Describe the two situations in which congestion can occur.

- What can a receiving host do to prevent data loss when data arrives too quickly ?

- Describe the three way handshake ( draw a diagram to show your understanding) and explain why it is important.

- What is meant by “sequence numbers are not based on a global clock in the network “ ?

- Explain “windowing” as used in flow control.

- Give an example of how a window size of 3 packets is negotiated down to 2 packets.

- What is forward reference acknowledgment ?

- Describe TCP as a connection oriented transport layer protocol and list some protocols that use TCP.

- TCP encapsulates data into segments. List and describe the fields in a TCP data segment.

- Describe UDP as a connectionless transport layer protocol and list some protocols that use UDP

- UDP encapsulates data into segments. List and describe the fields in a UDP data segment.

- What is the difference between hardware ports and software ports?

- Describe TCP and UDP port numbers and how they are used.

- List some well known reserved port numbers in TCP and UDP.

- Describe the Application Layer of the TCP/IP model.

- Describe the DNS system

- Compare and contrast the FTP and TFTP protocols and why is there a need for both ?

- Describe the hypertext transfer protocol HTTP.

- Describe the SMTP protocol.

- Describe SNMP used in the management of network devices.

- Describe the  features of Telnet.

Answer True or False for the following :

- The transport layer establishes a logical connection between two endpoints of a network.

- Protocols in the transport layer segment and reassemble data sent by upper-layer applications into the same transport layer data stream.

- This transport layer data stream provides end-to-end transport services.

- TCP is a connectionless protocol. TCP requires a connection to be established before data transfer begins

- UDP is a connection-oriented protocol that requires a connection to be established before data transfer begins.

- Data loss may occur if a host cannot process data as quickly as it arrives which forces the host to discard the data

- To provide flow control, TCP allows the source and destination hosts to communicate. The two hosts then establish a data-transfer rate that is agreeable to both.

- Synchronization occurs through an exchange of segments that carry a synchronize (SYN) control bit and the initial sequence numbers.

- During transmission , segments that are not acknowledged within a given time period will result in a retransmission

- The session, presentation, and application layers of the OSI model are bundled into the Transport layer of the TCP/IP model

- TFTP is a connection oriented service that uses User Datagram Protocol (UDP).

- FTP is a reliable, connectionless service that uses TCP to transfer files between systems that support FTP

- URL is an acronym for Uniform Resource Locator.

- The SMTP protocol transports email messages in ASCII format using TCP.

- SNMP uses UDP as its transport layer protocol.