Module 8 Ethernet Switching – Review Questions

- Explain to your colleague the operation of a bridge.

- What is a Content Addressable Memory table (CAM ) as it relates to a switch ?

- What is a microsegment as it refers to a switch ?

- Explain how there is no contention and therefore no collision on the media when a host is connected to a switch port.

- Which two technological advances made switches possible ?

- What is latency and what are some of the causes of latency ?

- Describe cut-through, store and forward and fragment free switching ?

- What type is used for asymmetric switching and what type of switching method is required for it ?

- Explain what is meant by a switch loop when multiple switches are connected

- How is the problem of potential switching loops overcome ?

- Explain each of the following terms. Where do collisions occur ?

Shared media environment –

Extended shared media environment –

Point-to-point network environment –

- What is a collision domain ? What devices separate collision domains ?

- Explain the 5-4-3-2-1 rule,  late collisions and consumption delay.

- What is a broadcast address ?

- Explain broadcast radiation and broadcast storms.

- What are the 3 sources of broadcasts in IP networks ?

- Explain the behaviour of each of the broadcasts in he previous question.

- Describe how Layer 1, Layer 2 and Layer 3 devices handle data movement.

- Define the term “segment”

Answer True or False for the following :

- In a network of 20 nodes, 1 collision domain exists if each node is plugged into its own switch port.  (F)

- A bridge divides a collision domain into smaller collision domains (T)

- A bridge and a switch work at Layer 2 using MAC addresses (T)

- Bridges and Switches learn about host port connections by listening to frame source addresses (T)

- Latency is the delay between the time a frame begins to leave the source device and when the whole frame reaches its destination (F)

- How a frame is switched to the destination port is a trade off between latency and reliability (T)

- When cut-through packet switching is used, the source and destination ports must have different bit rates to keep the frame intact (F)

- When asymmetric packet switching is used, store and forward mode must be used in the switch  (T)

- Layer 2 devices are the only devices that contain broadcasts (F)