Module 7 Ethernet Technologies– Quiz 2

- What is the name of the method used in Ethernet that explains how Ethernet works ?

  - TCP/IP

  - CSMA/CD

  - CMDA/CS

  - CSMA/CA

- What is the maximum distance for thick Ethernet without using a repeater ?

  - 185 m

  - 250 m

  - 500 m

  - 800 m

- 10 Mbps Ethernet operates within the timing limits offered by a series of no more than                           segments separated by no more than                          repeaters ?

  - Three, two

  - Four, three

  - Five, Four

  - Six, five

- Fast Ethernet supports up to what transfer rate ?

  - 5 Mbps

  - 10 Mbps

  - 100 Mbps

  - 1000 Mbps

- Identify  two Gigabit Ethernet cable specifications

  - 1000BASE-TX

  - 1000BASE-FX

  - 1000BASE-SX

  - 1000BASE-LX

  - 1000BASE-X

- What is the transmission medium for 1000BASE-SX ?

  - Long wave laser over single mode and multimode fiber

  - Category 5 UTP copper wiring

  - Balanced, shielded, 150-ohm, two pair STP copper cable

  - Short wave laser over multimode fiber

- 4D-PAM5 encoding method is used in which of the following Gigabit Ethernet ?

  - 1000BASE-LX

  - 1000BASE-SX

  - 1000BASE-T

  - 1000BASE-CX

- What is the IEEE standard for 10Gb Ethernet ?

  - 802.3z

  - 802.3u

  - 802.3ae

  - 803.3

- Which of the following is a feature of full-duplex transmission?

  - It offers two 10-to 1-Gbps data transmission paths

  - It doubles bandwidth between nodes

  - It provides collision – free environment

  - All of the above

- Which of the following is true concerning a bridge and its forwarding decisions ?

  - Bridges operate at OSI Layer 2 and use IP addresses to make decisions .

  - Bridges operate at OSI Layer 3 and use IP addresses to make decisions.

  - Bridges operate at OSI Layer 2 and use MAC addresses to make decisions .

  - Bridges operate at OSI Layer 3 and use MAC addresses to make decisions

Answers

1. B

2. C

3. C

4. C

5. C , D

6. D

7. C

8. C

9. D

10. C