Module 7 Spanning Tree Protocol  – Review Questions

- Describe redundancy and explain why it is desirable.

- What are some of the problems that can occur with redundancy in networks ?

- How does the spanning tree protocol help to overcome the problems associated with redundancy ?

- Explain the concept and benefits of redundant topologies.

- Describe how physical loops in topologies can cause broadcast storms

- Describe an example of a broadcast storm.

- Describe how in a redundant topology multiple frames can arrive at an end host.

- Describe how Media Access Database instability can occur in a redundant topology.

- Describe briefly the main feature of the Spanning Tree Protocol.

- Describe how STP can by used to calculate a loop free network.

- Complete the following table

| Link Speed | Cost (IEEE spec) |
| --- | --- |
| 10 Gbps |  |
| 1 Gbps |  |
| 100 Mbps |  |
| 10 Mbps |  |

- Describe the state of a converged network in the STP algorithm.

- Describe the process of electing a root bridge.

- Describe each of the following fields in the Bridge Protocol Data Unit (BPDU) .

Root BID ---

Root Path Cost --- 
Sender BID --- 
Port ID ---

- Describe the 5 port states of a switch that used STP.

- When conditions must be met for a network to have converged ?

- Briefly describe the Rapid Spanning Tree Protocol RSTP

Answer True or False for the following :

- Multicasts are treated as broadcasts by the switches. Broadcast and multicast frames are flooded out all ports, except the one on which the frame was received.

- An example of a broadcast would be an ARP request for the Layer 2 address of a router.

- In the Layer 2 header, there is no Time To Live (TTL) value. If a frame is sent into a Layer 2 looped topology of switches, it can loop forever

- The Spanning Tree Algorithm can take a relatively long time to converge

- A new algorithm called the rapid spanning-tree algorithm was developed to reduce the time for a network to compute a loop free logical topology.

- Ethernet bridges and switches can implement the IEEE 802.1d Spanning-Tree Protocol

- BPDUs are not received on blocked ports

- When a switch first starts up, it assumes it is the root switch and sends BPDUs that contain the switch MAC address in both the root and sender BID

- Network administrators can not set the switch priority to a smaller value than the default, to make the BID smaller.